Quick guide to use M5 RGB LED

- Download and use the latest libraries from Github repo (or manually install AdaFruit_NeoPixel library in Arduino IDE).

- Connect the LED module:

Use this cable

Connect the free-end (Black) connector cable to the Arduino Board using the provided gold-pin header.

Connect the other end (white) of the connector cable to the LED Module (Take note to use the socket nearest to the module mounting holes).

- Run the sample program ( a copy is in github  -  lab4: rgb_led_simple):

#include <Adafruit_NeoPixel.h>

// Which pin on the Arduino is connected to the NeoPixels?

#define PIN        6 // On Trinket or Gemma, suggest changing this to 1

// How many NeoPixels are attached to the Arduino?

#define NUMPIXELS  3 // M5 RGB LED has only 3 LEDs

// When setting up the NeoPixel library, we tell it how many pixels,

// and which pin to use to send signals. Note that for older NeoPixel

// strips you might need to change the third parameter -- see the

// strandtest example for more information on possible values.

Adafruit_NeoPixel pixels(NUMPIXELS, PIN, NEO_GRB + NEO_KHZ800);

void setup()

{

pixels.begin(); // INITIALIZE NeoPixel strip object (REQUIRED)

}

void loop()

{

pixels.clear();   // Set all pixel colors to 'off'

pixels.show();    // Send the updated pixel colors to the hardware.

delay(2000);      //

pixels.setPixelColor(0, pixels.Color(255, 0, 0));   // LED 0 - Red

pixels.setPixelColor(1, pixels.Color(0, 255, 0));   // LED 1 - Green

pixels.setPixelColor(2, pixels.Color(0, 0, 255));   // LED 2 - Blue

pixels.show();                                      // Send the updated pixel colors to the hardware.

delay(2000);

}